- Paint shop Plant

- Powder coating plant

- Pollution control Equipment’s

- Paint shop Plant

- Cleaning Room

- Buffing Booth

- Paint Booth

- Flash Off Zone

- Paint Drying Room / Oven

Cleaning Room

Design Calculation

Exhaust Air Volume : Area x Velocity

Inlet Air volume : Exhaust air volume x 10% add

Material weight  : Total Surface area (5 Sides – Botto side neglected)

Buffing Booth

Type : Down Draft / Side Draft

Design Calculation

Exhaust Air Volume : Area x Velocity

Inlet Air volume : Exhaust air volume x 10% add

Material weight  : Booth Total Surface area (5 Sides – Botto side neglected) + Suction chamber total area (5 Sides – Botto side neglected)

Paint Booth

Type : Down Draft / Side Draft / Cross Draft

If side Draft -

Design Calculation

Exhaust Air Volume : Area x Velocity

Inlet Air volume : nil

Material weight  : Booth Total Surface area (5 Sides – Botto side neglected) + Suction chamber total area (5 Sides – Botto side neglected)

If Cross Draft -

Design Calculation

Exhaust Air Volume : Area x Velocity

Inlet Air volume : Exhaust air volume x 10% less

Material weight  : Booth Total Surface area (5 Sides – Botto side neglected) + Suction chamber total area (5 Sides – Botto side neglected)

If Down Draft -

Design Calculation

Exhaust Air Volume : Area x Velocity

Inlet Air volume : Exhaust air volume x 10% less

Material weight  : Booth Total Surface area (5 Sides – Botto side neglected) + Suction chamber total area (5 Sides – Botto side neglected)

Flash off zone

Type : Down Draft / Side Draft

Design Calculation

Exhaust Air Volume : Area x Velocity

Inlet Air volume : Exhaust air volume x 10% add

Material weight  : Total Surface area (5 Sides – Botto side neglected)

Paint Drying Room / Oven

Type  : Room

Design Calculation

Exhaust Air Volume : Volume x ACH

Material weight  : Total Surface area (5 Sides – Botto side neglected)

Type  : Oven

Design Calculation

Exhaust Air Volume : Volume x ACH

Heat load : volume (ft3) / 100

100 ft3 = 12 kW

kW x 860 = kCal

Material weight  : Total Surface area (5 Sides – Botto side neglected)